NanoIO

Authors:

David Freese, W1HKJ

Ervin Hegedüs, HA2OS

An FSK / CW interface based on the Arduino nano.  Sketch is expanded version of

tinyFSK, http://www.frontiernet.net/~aflowers/tinyfsk/

and includes an FSK modulator, a CW computer interface, a CW iambic-A/B keyer.

FSK Specifications:

- 5 bit Baudot

- baud rates 45.45, 50, 75 and 100

CW Specifications:

- 5 to 100 WPM

- dash/dot ratio adjustable 2.5 to 3.5

- in-line increment decrement WPM using ^ and | characters

- incremental size user adjustable

PTT signal generated by Arduino

Both: an internal buffer of 200 characters is available for buffered transmit.

Hardware requirement

- Arduino nano or compatible (author used nano from Elegoo)

- LTV-847 quad opto-isolator

- 4 620 ohm ¼ watt resistor (should work with 500 < R < 820 ohm)

- suitable connectors to interface to transceiver

Default pin assignments defined in Arduino Sketch for the above circuit.

D9 / PIN 9 – spare

D10 / PIN 10 – PTT

D11 / PIN 11 - FSK_PIN

D12 / PIN 12 – CW

When compiling for MORTTY, you should change the pin assignments:

D11 / PIN 11 – common CW / FSK output

D13 / PIN 13 – PTT output

Default startup configuration:

- nanoIO 1.0

- Mode: FSK

- FSK: Baud: 45.45, Mark LOW

- CW: WPM: 18/18, dash/dot 3.00, incr 2, IambicA keyer

This is the current command set / configuration parameters for nanoIO

USB serial command strings all begin with the tilde, ~, character

Cmd ~...

- C,c CW mode

- F,f FSK mode

- T,t CW Tune

- Snnns computer wpm 10...100

- Unnnu key (user) wpm 10...100

- Dnnnd dash/dot 250...350 (2.5...3.5)

- In CW incr (1..9)

- A,a IambicA

- B,b IambicB

- K,k Straight key

- 0 FSK mark = HIGH

- 1 FSK mark = LOW

- 4 45.45 baud

- 5 50 baud

- 7 75 baud

- 9 100 baud

- ? Show config

- W Write EEPROM

- ~ Show cmds

In line special characters:

Both:

- [ PTT on

- ] PTT off

- \ escape; clear internal buffer and set PTT off

CW:

- ^ increase computer wpm by increment value (default is 2) *

- | decrease computer wpm by increment value *

- % <SK>

- & <AS>

- + <KN>

- - <BT>

- < <AS>

- = <BT>

- > <AR>

- { left brace <HM>

- } right brace <VE>

* - the ^| wpm modifiers do not effect the paddle CW, just the computer CW.

for example, a CW string might contain:

- ~C~S24s[tu ^^5nn|| k]

- Mode is changed to CW

- Computer WPM is set to 24

- PTT is set ON

- the string “tu “ is sent

- Computer WPM is increased by 2 increment factors

- the string “5nn” is sent

- Computer WPM is decreased by 2 increment factors

- the string “ k” is sent

- PTT is set OFF

PTT will enable before starting the CW transmission.  PTT will disable after the last CW character (k) is completed.

nanoIO is a merger of my own code and:

tinyFSK, by Andrew T. Flowers K0SM
Iambic Keyer, by Steven T. Elliott

It expands the tinyFSK to include 100 WPM Baudot TTY. It shares a common set of control strings. The default wake up mode is FSK.

The keyer implementation is less robust than Steven's Iambic Keyer. It only provides Straight key, Iambic-A, and Iambic-B modes. It does add weighting to the key output. The key weighting and the computer CW weighting are the same. The current implementation does not have a tone output.

CW is generated from USB serial input strings and/or the paddle inputs. Paddle input overrides the serial string, but does not currently clear the string buffer. That could be changed, but the USB serial interface provides a very easy escape character to do just that.

NanoIO can be used as the sketch for the MORTTY (c) hardware interface.  MORTTY shares a common output pin for both CW and FSK keyline. The user will have to swap FSK / CW line connectors when changing mode.